and I want the following to be implemented as well:

ignore same-seq updates (even if better hop).

“When installing a route from neighbor, store metric = receivedMetric + 1; advertise stored metric.”

Add random jitter to periodic incremental/full timers (few hundred ms to a couple seconds) to prevent synchronization collisions.

Full dumps can exceed LoRa frame size. Add a requirement:

If full dump doesn’t fit, split into chunks with (chunkId, totalChunks) or send only a bounded subset per period.

to detect link break: Each node:

Periodically hears routing broadcasts (incremental / full dumps) from its neighbors.

Maintains a last-heard timestamp per neighbor.

A link to neighbor N is considered broken if:

now - lastHeard[N] > neighborTimeout

Where:

neighborTimeout is typically 2–3× the incremental update period.

now give me the full dsdv implementation steps

---

Acknowledged. Plan: make DSDV selectable in INI, wire NED params, then implement DSDV state, timers, neighbor tracking, update send/receive, and logging in LoRaNodeApp, preserving legacy when not selected. Settling time is disabled; unreachable updates use odd/newer seq with infinite metric; equal-seq updates are ignored.

INI Configuration

Add protocol selector and parameters:

**.routingProtocol: "legacy" | "dsdv"

**.dsdvIncrementalPeriod: cadence for incremental updates (e.g., 15s)

**.dsdvFullUpdatePeriod: cadence for full dumps (e.g., 120s)

**.dsdvTriggeredMinInterval: min gap for triggered updates (e.g., 3s)

**.dsdvRouteLifetime: route GC timeout (e.g., 600s)

**.dsdvTimerJitterMin, **.dsdvTimerJitterMax: jitter range (e.g., 0.2s..2s)

**.dsdvNeighborTimeout: if omitted, derive as k * dsdvIncrementalPeriod with k≈2.5

**.dsdvMaxEntriesPerPacket: cap entries per routing packet (for LoRa frame size)

**.dsdvUseChunking: true|false to choose chunking vs. bounded subset

Provide a DSDV-enabled sample config; keep existing configs with routingProtocol="legacy" unchanged.

NED Parameters

Add and pass these params into LoRaNodeApp submodules in:

LoRaNode.ned

endNode.ned

endRescueNode.ned

Ensure LoRaEndNodes (which map to LoRaNodeApp via your package mapping) receive these params.

State and Data Structures (LoRaNodeApp)

Extend routing entry:

destId, nextHop, metric, seqNum (uint32), valid (bool), installTime (SimTime)

Node-local:

ownSeqNum (uint32) for self-originated destination; even when valid

changedSet (destIds) to drive incremental ads

lastHeard[neighborId] map for neighbor timestamps

lastTriggeredUpdateTime for debounce

Constants/params:

INFINITE_METRIC sentinel; neighborTimeout value

Message Format

Routing update entry (DSDV mode):

{destId, seqNum, metric, flags} where flags includes valid/invalid

Full-dump chunking header (if chunking enabled):

{chunkId, totalChunks, sequenceTag} plus entries

Legacy mode:

Preserve existing beacon format.

Initialization

Read routingProtocol and all DSDV params.

If dsdvNeighborTimeout not set, compute dsdvNeighborTimeout = neighborTimeoutFactor * dsdvIncrementalPeriod (factor ~ 2.5).

Schedule periodic timers with jitter:

Incremental: next = now + dsdvIncrementalPeriod + U[jitterMin, jitterMax]

Full: next = now + dsdvFullUpdatePeriod + U[jitterMin, jitterMax]

Do not schedule settling timers (explicitly disabled).

Neighbor Tracking and Link-Breaks

On receiving any routing broadcast (incremental or full) from neighbor N:

Update lastHeard[N] = now

Periodic neighbor check (align with incremental timer or separate):

For each N, if now - lastHeard[N] > neighborTimeout: mark link to N broken

For each route whose nextHop == N:

Set metric = INFINITE_METRIC, valid = false

Compute unreachableSeq = lastKnownSeq(dest) + 1 (odd/newer); set seqNum = unreachableSeq

Add dest to changedSet

If any invalidations occurred and now - lastTriggeredUpdateTime > dsdvTriggeredMinInterval, schedule/send a triggered incremental update; set lastTriggeredUpdateTime = now

Sending Updates (DSDV mode)

Incremental updates:

Build entries only for changedSet; for each entry:

Advertise metric = storedMetric (already includes “+1 on install” via receive path)

Include destId, seqNum, metric, validFlag

Clear changedSet after send

Full dumps:

Iterate over entire table and build entries

If packet would exceed LoRa payload (use dsdvMaxEntriesPerPacket or MTU), then:

If dsdvUseChunking:

Partition entries into chunks of size ≤ max; send chunk packets with {chunkId, totalChunks}

Else (bounded subset):

Send the next window of up to dsdvMaxEntriesPerPacket entries; rotate window each period

Triggered updates:

Sent immediately (broadcast) when invalidations or new higher-seq info appears, but enforce dsdvTriggeredMinInterval

Legacy mode:

Keep current sendRoutingPacket() logic unmodified.

Receiving Updates (DSDV mode)

For each received entry (D, seq, met, flags) from neighbor N:

Compute installed metric: installedMetric = met + 1

If seq > table[D].seqNum:

Install: nextHop = N, metric = installedMetric, seqNum = seq, valid = !invalidFlag, installTime = now

Add D to changedSet

Else if seq == table[D].seqNum:

Ignore same-seq updates entirely (even if hop count is better)

Else (seq < current):

Ignore

If an entry is invalid (invalidFlag true or met == INFINITE_METRIC):

Apply above acceptance rules; if accepted, invalidate locally and add to changedSet

Legacy mode:

Preserve existing processing.

Routing Table Maintenance

Garbage collection:

Periodically (e.g., at each incremental tick), remove or keep invalid entries older than dsdvRouteLifetime

Optionally keep invalid entries with latest odd seqNum to prevent re-accepting older info

Self entry:

Maintain own destination entry with seqNum = ownSeqNum (even). When local conditions change requiring invalidate (rare for self), increment to odd; when revalidating, bump to next even.

Logging/Diagnostics

Extend snapshots in LoRaNodeApp.cc:

Include columns: destId, nextHop, metric, seqNum, valid, installTime

If chunking active, include chunkId/totalChunks in a send log row or a separate control log

Keep existing delivered path logs; optionally add seqNum at the time of forwarding.

Backward Compatibility

All DSDV behavior is gated behind routingProtocol == "dsdv".

When routingProtocol == "legacy", current fields, timers, and behaviors remain as-is.

Testing and Tuning

DSDV basic:

Static topology: confirm tables converge; verify only higher seqNum changes are accepted; same-seq better hops are ignored

Link break:

Induce neighbor loss; verify invalidation: seq = old + 1 (odd), metric = ∞, triggered update sent after debounce

Fragmentation/subset:

Increase dsdvMaxEntriesPerPacket low to force splitting; confirm chunk sequence or window rotation across periods

Jitter:

Inspect send times to verify randomized offsets around base periods

Legacy parity:

Run existing configs with legacy; CSVs should match pre-DSDV behavior

If you want, I can now draft the exact parameter names/defaults to add in the NED files and point to the specific methods in LoRaNodeApp to implement each step, still without changing files.

---

Step-by-step implementation

  - Protocol switch (INI)

Add once, to enable/disable DSDV without code path risk.

INI params:

**.routingProtocol = "legacy" | "dsdv"

**.dsdvIncrementalPeriod = 15s

**.dsdvFullUpdatePeriod = 120s

**.dsdvTriggeredMinInterval = 3s

**.dsdvRouteLifetime = 600s

**.dsdvTimerJitterMin = 0.2s

**.dsdvTimerJitterMax = 2s

**.dsdvNeighborTimeout = 40s (or compute ≈ 2.5 × incremental)

**.dsdvUseChunking = true|false

**.dsdvMaxEntriesPerPacket = 10 (tune based on payload)

Add a new config that sets routingProtocol = "dsdv"; keep all existing configs as "legacy".

2. Wire NED parameters

Add/pass these parameters into the LoRaNodeApp submodule for every node type that embeds it (remember your package.ned maps EndNodes to LoRaNodeApp):

Files to edit:

LoRaNode.ned

endNode.ned

endRescueNode.ned

LoRaNodeApp.ned — declare new params at module level (default values).

Confirm the params appear in your .ini via module path resolution for all relevant nodes.

3. Extend routing packet format (message definition)

Update LoRaAppPacket.msg:

Add DSDV fields to each advertised entry:

Extend LoRaRoute or add a DSDV-specific route entry: {destId, seqNum (uint32), metric (int), flags (uint8)}.

Keep priMetric/secMetric for legacy; in DSDV, use priMetric for metric, and add a dedicated seqNum (don’t overload secMetric).

Optionally add a full-dump header for chunking:

{fullDumpId (uint32), totalChunks (uint16), chunkId (uint16), entryCount (uint16)}.

Rebuild will regenerate LoRaAppPacket_m.h automatically.

Note: If you want zero message layout risk, add a separate message/array for DSDV updates rather than overloading LoRaRoute.

Extend routing table structure (DSDV state)

In LoRaNodeApp.h:

Extend singleMetricRoute with:

uint32_t seqNum;

bool valid; (separate from valid timestamp — your valid field today is simtime_t valid; rename or add isValid to avoid confusion)

simtime_t installTime;

Add node-local:

uint32_t ownSeqNum = 0;

std::unordered_set<int> changedSet;

std::unordered_map<int, simtime_t> lastHeard;

simtime_t lastTriggeredUpdateTime = 0;

Define const int INFINITE_METRIC = <large_value>; (e.g., 0x3FFF; must fit LoRaRoute.priMetric).

Keep dual-metric table untouched unless you plan DSDV for it too.

Initialization and parameters

In LoRaNodeApp.cc, initialize(int stage):

Read routingProtocol and new DSDV params.

If DSDV:

Schedule periodic incremental and full-dump timers using jitter:

Next incremental = base + uniform(jitterMin, jitterMax)

Next full = base + uniform(jitterMin, jitterMax)

Compute neighborTimeout if not supplied (≈ 2.5 × incremental).

Legacy: leave current timers as-is.

Neighbor tracking and link-break detection

In manageReceivedRoutingPacket(cMessage*):

On any routing packet from neighbor N, set lastHeard[N] = simTime().

Add a periodic neighbor sweep (can be at incremental tick):

For each N, if simTime() - lastHeard[N] > neighborTimeout, mark link-down:

For every route with nextHop == N:

Set metric = INFINITE_METRIC, isValid = false

Set seqNum = lastKnownSeq + 1 (odd/newer)

Add dest to changedSet

If simTime() - lastTriggeredUpdateTime > dsdvTriggeredMinInterval, send a triggered incremental and update lastTriggeredUpdateTime.

DSDV receive path (ignore same-seq updates)

In manageReceivedRoutingPacket(cMessage*) under DSDV branch:

For each entry (D, seq, met, flags) from neighbor N:

Compute stored metric as installed = met + 1 (store this, and advertise exactly your stored metric later).

If seq > table[D].seqNum: accept, set nextHop=N, metric=installed, seqNum=seq, isValid = !invalidFlag, installTime=now, and add D to changedSet.

Else if seq == table[D].seqNum: ignore (even if metric is better) — your explicit rule.

Else: ignore (older info).

If entry is invalid (flags or INFINITE_METRIC), apply the same seq comparison rules; if accepted, keep it invalid and add to changedSet.

DSDV send path (incremental / full / triggered)

Gate sendRoutingPacket():

Legacy → current behavior.

DSDV:

Incremental:

Build entries from changedSet only, using stored metric (already “+1 on install”).

Include seqNum and valid flag; clear changedSet after successful send.

Full-dump:

Build a list of all current entries.

If exceeds payload:

If dsdvUseChunking:

Split into chunks: compute maxEntriesPerPacket from routingPacketMaxSize or dsdvMaxEntriesPerPacket.

Send packets with {fullDumpId, totalChunks, chunkId, entryCount}.

Optionally spread chunks over short jittered gaps to reduce collisions.

Else (bounded subset):

Send up to dsdvMaxEntriesPerPacket entries per full period; rotate the window next period.

Triggered:

On invalidation or new higher seq info, send incremental immediately (debounced by dsdvTriggeredMinInterval).

Chunk reassembly (if enabled)

Receiver:

On a full-dump chunk:

Maintain reassembly state by fullDumpId: track totalChunks, received bitmap, and store entries.

Optionally process entries immediately; or apply atomically on all-chunks-received.

Discard incomplete reassembly after dsdvChunkReassemblyTimeout (e.g., 2 × incremental).

Sender pacing:

Add small gap and jitter between chunks to avoid collision and duty-cycle spikes.

Logging and diagnostics

In logRoutingSnapshot():

Add seqNum, isValid, and installTime columns when routingProtocol=="dsdv".

Optionally add a separate log line when sending chunked full-dumps (include fullDumpId, chunkId, totalChunks).

Backward compatibility

All DSDV code paths are strictly gated by routingProtocol == "dsdv".

Validate that when using "legacy", snapshots and forwarding behavior remain unchanged.

Testing and tuning

New DSDV config:

Enable DSDV; run a small, mobile scenario.

Verify:

Same-seq packets are ignored; only higher-seq accepted.

Stored metric = received + 1; advertised stored metric.

On link-break, routes are invalidated with seq = last + 1 (odd/newer), metric = ∞, and a (debounced) triggered update goes out.

Full-dump chunking or subset works without exceeding payload; if chunking, reassembly completes or degrades gracefully.

Jitter is applied to periodic timers (no global sync bursts).

Legacy parity:

Run a known scenario with routingProtocol="legacy"; compare CSVs to baseline.

Concrete code touchpoints

Message schema:

LoRaAppPacket.msg — add DSDV entry fields and optional chunk header.

NED:

LoRaNodeApp.ned — declare new params.

LoRaNode.ned, endNode.ned, endRescueNode.ned — pass params into submodule.

App logic:

LoRaNodeApp.h — new fields in singleMetricRoute and new members (ownSeqNum, lastHeard, etc.).

LoRaNodeApp.cc:

initialize(int stage) — read params, schedule timers (with jitter).

handleMessage(cMessage*) / handleSelfMessage(cMessage*) — drive periodic incremental/full and neighbor sweeps.

sendRoutingPacket() — gate legacy vs DSDV; implement incremental/full/triggered; chunking/subset.

manageReceivedRoutingPacket(cMessage*) — DSDV receive rules (higher-seq only; same-seq ignored; store=recv+1).

Neighbor timeout sweep — invalidate routes (odd/newer seq, ∞ metric), debounced trigger.

logRoutingSnapshot() — include seqNum/isValid for DSDV.

Optional: reuse routingPacketMaxSize to compute dsdvMaxEntriesPerPacket, or keep it separate.

---

Here’s the clean step-by-step list, in the order you should implement it. You can paste this directly into Copilot or use it as your checklist.

Step-by-step implementation list (E-DSDV–style, no settling time)

- Add protocol switch (INI)

  - Add routingProtocol = "legacy" | "edsdv".

  - Keep all existing configs as "legacy".

  - Create one new config enabling "edsdv".

- Add DSDV/E-DSDV parameters (INI)

  - dsdvIncrementalPeriod

  - dsdvFullUpdatePeriod

  - dsdvTriggeredMinInterval

  - dsdvRouteLifetime

  - dsdvTimerJitterMin, dsdvTimerJitterMax

  - dsdvNeighborTimeout (or derive ≈ 2.5 × incremental)

  - dsdvUseChunking

  - dsdvMaxEntriesPerPacket

- Wire parameters in NED

  - Declare new params in LoRaNodeApp.ned.

  - Pass them into LoRaNodeApp from:

    - LoRaNode.ned

    - endNode.ned

    - endRescueNode.ned

  - Verify they appear in .ini via module paths.

- Extend routing packet format

  - Update LoRaAppPacket.msg.

  - Add DSDV route entry fields:

    - destId, seqNum, metric, flags (valid/invalid).

  - Optionally add full-dump chunk header:

    - fullDumpId, totalChunks, chunkId, entryCount.

  - Rebuild to regenerate message headers.

- Extend routing table state

  - Add to routing entry:

    - seqNum, isValid, installTime.

  - Add node-local state:

    - ownSeqNum

    - changedSet

    - lastHeard[neighbor]

    - lastTriggeredUpdateTime

  - Define INFINITE_METRIC.

- Initialize E-DSDV in LoRaNodeApp

  - Read routingProtocol and params.

  - If "edsdv":

    - Schedule incremental + full-dump timers with jitter.

    - Compute neighborTimeout if not set.

  - If "legacy": leave existing behavior untouched.

- Track neighbors

  - On receiving any routing packet from neighbor N:

    - Update lastHeard[N] = simTime().

  - Periodically (each incremental tick):

    - Detect neighbors with now - lastHeard > neighborTimeout.

- Detect link breaks

  - For each timed-out neighbor N:

    - For every route using nextHop == N:

      - Set metric = INFINITE_METRIC

      - Set isValid = false

      - Set seqNum = newer (force odd if desired)

      - Add destination to changedSet

  - Trigger debounced incremental update.

- Receive routing updates (E-DSDV rules)

  - For each entry (D, seq, met, flags) from neighbor N:

    - If seq > table[D].seqNum:

      - Install route: nextHop=N, metric=met+1, seqNum=seq, isValid

      - Add D to changedSet

    - Else if seq == table[D].seqNum:

      - Ignore completely (even if hop count is better)

    - Else:

      - Ignore

    - If entry is invalid:

      - Apply same seq comparison; keep invalid if accepted.

- Send routing updates

  - Gate all logic by routingProtocol.

  - Incremental updates:

    - Send only destinations in changedSet.

    - Clear changedSet after send.

  - Full dumps:

    - Send entire table periodically.

    - If too large:

      - Chunk (preferred) or

      - Send bounded subset / rotating window.

  - Triggered updates:

    - Send immediately on invalidation or higher-seq info.

    - Enforce dsdvTriggeredMinInterval.

- Chunking (if enabled)

  - Split full dump into packets of ≤ dsdvMaxEntriesPerPacket.

  - Add {fullDumpId, totalChunks, chunkId} to each.

  - Add small inter-chunk gap + jitter.

  - Receiver:

    - Process entries immediately (best-effort), or

    - Reassemble until all chunks arrive (optional).

- Route lifetime management

  - Periodically remove or age out entries older than dsdvRouteLifetime.

  - Optionally retain invalid entries with newest seqNum to prevent rollback.

- Logging and debugging

  - Extend routing snapshot logs:

    - destId, nextHop, metric, seqNum, isValid, installTime.

  - Log chunk send events (optional).

- Backward compatibility check

  - Confirm "legacy" mode:

    - Uses old packet format

    - Uses old timers

    - Produces identical CSV/log output.

- Testing and validation

  - Static test: tables converge; seqNums never decrease.

  - Mobility test: link breaks → invalidations → triggered updates.

  - Chunk test: no packet exceeds payload; chunks rotate correctly.

  - Legacy parity: results unchanged when "legacy".

If you want, next I can:

- Condense this into a one-page implementation checklist

- Rewrite it as a Copilot task prompt

- Or map each step to exact functions in LoRaNodeApp.cc

Just say which one.